Marker Selection Process Flow

A structured workflow for selecting a commercially available marker that fits the project's substrate, detection needs, and regulatory boundaries using AI tools, supplier catalogs, and technical literature.

Phase 0: Project Inputs (Given)

These are defined by the project and are not part of the selection, they frame it.

- Target substrate: The material to be marked (e.g., metal, plastic, oil, fuel, textile, paper, coating), as defined by the project scope.

- Regulatory constraints. The compliance boundaries that apply to the project's industry, region, and end-use (e.g., food contact, REACH, RoHS)

Everything that follows must respect these two inputs.

Phase 1: Define the Marker Form

The marker will always be one of:

- A metal element,

- A metal compound (oxide, carbonate, sulfate, chloride etc.), or

- An organocomplex carrying the metal.

The chosen form must match the substrate's physical state (e.g., oil/fuel → oil-soluble organocomplex; solid polymer → oxide or salt; coating → dispersible compound).

Phase 2: Initial Candidate Discovery & AI Screening

- Build the AI query. Combine substrate + form + functional requirements + regulatory framework into a single targeted prompt.

  - Example: "List market-available [organocomplexes / metal compounds] fully soluble in [substrate], inert and stable in it, with zero impact on performance, compliant with [regulatory framework]." (please see appendix 1 for a conversation example)

- Receive a candidate list of metals and their commercially available molecular forms (e.g., neodecanoates, octoates, 2-ethylhexanoates, oxides, carboxylates).

- Application-dependent constraints. Filter against thermal stability, optical requirements (color/haze/UV), toxicology and exposure limits, processing compatibility (shear, UV, humidity), end-of-life behavior, and other constraints, specifically per project.

- Map structural and formulation variations. The same metal can be delivered through different carriers/ligands (e.g., Zr neodecanoate vs. Zr octoate vs. Zr(TMHD)₄), and these variations drive solubility, stability, and XRF signal cleanliness.

Phase 3: Tiered Categorization

Sort the AI-generated candidates into three tiers (sorting is already done using AI), this is the key decision-making layer:

| Tier | Meaning | Typical content |
| --- | --- | --- |
| A — Strong candidates | Best fit: oil/substrate soluble, stable, low impact, commercially available, low XRF background | Primary shortlist |
| B — Analytically excellent, needs validation | Clean XRF signal, unique forensic value, but limited application history in the substrate | Secondary shortlist / forensic layer |
| C — Not recommended | Already present in the substrate's normal additive package, wear metals, regulated elements, or known interference | Exclude |

Phase 4: Regulatory Dosing Window

- Translate regulations into ppm limits. Standards rarely list "approved markers"  instead they cap finished-product properties (e.g., phosphorus, sulfur, sulfated ash, viscosity, deposits).

- Set a recommended ppm range per candidate that stays well inside those limits while remaining above the XRF detection floor (typically a few ppm to tens of ppm for metals). Chen: this one usually underestimates the required ppm level, hopefully can be improved by providing previous data/XRF spectra/device model etc.

- Build a single summary table- for example: element, market-available molecule, recommended ppm, regulatory/technical fit, recommendation.

Phase 5: Background & Interference Check

This step could happen at the beginning of the project, it depends if we already received the target material, as we’ll need to measure it using the XRF.

- Check natural background of each candidate element in virgin and used substrate samples.

- Check XRF spectral overlap with elements already present in the substrate's normal additive package or contamination profile.

- If a candidate fails → return to Phase 2 with the exclusion added to the AI query ("exclude Zr due to background") and regenerate the shortlist.

- Repeat until the surviving candidates have a clean, distinguishable XRF signal at the target ppm.

Phase 6: Commercial Availability Audit

- Off-the-shelf only. No custom synthesis, too slow, expensive, and regulatory-heavy. Only standard catalog products are considered.

- Cross-reference supplier catalogs (Sigma-Aldrich, Merck, Strem Chemicals, specialty organometallic vendors etc.). For each candidate, verify exact molecular form, metal content, purity grade, particle size, physical form, available volume, lead time, and SDS / hazard classification.

- Drop anything not available or not application-fit. Output: a clean, sourceable, application-fit shortlist.

Phase 7: Performance & Literature Evaluation

- Validate in the substrate matrix: solubility, stability over time, processing compatibility, and zero/negligible impact on host performance.

- Pull supporting literature: datasheets, application notes, scientific articles via AI-assisted retrieval, to confirm real-world behavior and known limitations.

Phase 8: Final Selection

- Build the decision matrix across:

  - Metal / XRF detection fit

  - Substrate compatibility (solubility, stability, inertness)

  - Regulatory ppm window

  - Commercial availability

  - Cost and supply stability

- Document the rationale so the choice is traceable for the customer, regulators, and the internal team.

Appendix 1 – AI conversation example

deborah amsallem8:34 AM

i need to mark engine oil for shell with elemental molecules, fully dissolvable in engine oil, inert to engine oil, stable in engine oil , zero impact on engine oil performance, in full compliance with automotive engine oil . please provide me list of market available molecule that fits my request

ChatGPT8:34 AM

For a Shell engine-oil authentication program, I would separate the candidates into three categories:

Category A – Strong candidates (best fit for SMX-style marking)

These are fully oil-soluble organometallic compounds, used at very low ppm levels (typically 5–50 ppm metal), with minimal impact on viscosity, TBN, oxidation, volatility, detergency, wear, or emissions when properly dosed.

| Element | Molecule | Oil Solubility | Stability in Engine Oil | Automotive Compatibility | Comments |
| --- | --- | --- | --- | --- | --- |
| Zr | Zirconium neodecanoate | Excellent | Excellent | Very good | Probably the best replacement for Zr(TMHD)₄; significantly cheaper. |
| Zr | Zirconium octoate (2-EH) | Excellent | Excellent | Good | Lower cost than TMHD; commonly supplied as oil solution. |
| Zr | Zirconium carboxylate esters | Excellent | Excellent | Good | Commercially available as oil-soluble metal carboxylates. |
| Ti | Titanium neodecanoate | Excellent | Very good | Good | Similar chemistry to Zr carboxylates. |
| Ti | Titanium octoate | Excellent | Very good | Good | Low cost and highly soluble. |
| Mo | Molybdenum neodecanoate | Excellent | Excellent | Excellent | Mo is already used in many engine oils as friction modifier. |
| W | Organotungstate compounds | Excellent | Excellent | Good | Distinct elemental signature, low natural background. |

Category B – Excellent analytically, but requires validation

These provide very clean elemental signatures but are less common in commercial lubricant additive packages.

| Element | Molecule |
| --- | --- |
| Y | Yttrium neodecanoate |
| Yb | Ytterbium neodecanoate |
| La | Lanthanum neodecanoate |
| Ce | Cerium neodecanoate |
| Nd | Neodymium neodecanoate |
| Hf | Hafnium neodecanoate |

Advantages:

- Extremely low natural background in engine oils.

- Easy XRF/ICP differentiation.

- Unique forensic signature.

Disadvantages:

- Fewer automotive compatibility studies.

- Need oxidation, deposit, and catalyst poisoning assessment.

Category C – Not recommended

Although detectable, I would avoid them for OEM-facing programs:

| Element | Reason |
| --- | --- |
| Ca | Already present in detergent packages. |
| Mg | Already present in detergent systems. |
| Zn | Already present in ZDDP additives. |
| P | Already present in ZDDP additives. |
| B | Common additive chemistry. |
| Cu | Potential catalyst and oxidation concerns. |
| Fe | High engine wear background. |
| Ni | Regulatory concerns. |
| V | Catalyst poisoning concerns. |

If I were designing a Shell deployment today

Option 1 – Lowest technical risk

Zirconium neodecanoate

- Fully oil soluble

- Commercially available

- Low cost

- Similar behavior to your current Zr(TMHD)₄

- Can be dosed at 10–20 ppm Zr

- Very easy ICP and XRF detection

- Minimal formulation impact expected at trace levels.

Option 2 – Best forensic uniqueness

Ytterbium neodecanoate + Zirconium neodecanoate

- Zr = authentication

- Yb = anti-counterfeit layer

- Very low natural background

- Two-element ratio can create digital signatures

Option 3 – Most OEM-friendly

Molybdenum neodecanoate

- Mo chemistry is already accepted in many modern engine oils as a lubricant additive.

- Easier discussion with lubricant formulators.

What Shell's lubricant formulators will likely accept most easily

- Zirconium neodecanoate ⭐

- Molybdenum neodecanoate

- Titanium neodecanoate

- Ytterbium neodecanoate (for forensic traceability)

- Lanthanum neodecanoate

For your specific Shell PCMO program, I would continue with zirconium chemistry and evaluate:

- Zr(TMHD)₄ (current benchmark)

- Zirconium neodecanoate

- Zirconium octoate

These three molecules are the closest in behavior while remaining fully oil-soluble organometallic species and are the most realistic candidates for maintaining compatibility with API/ILSAC/ACEA engine oils at 10–20 ppm elemental concentration.

One important point: no supplier can legitimately claim "zero impact" on engine oil performance without testing. For Shell, the correct statement is that at 10–20 ppm elemental concentration the marker is expected to have negligible impact, which must then be confirmed through standard lubricant tests (viscosity, oxidation, foaming, air release, wear, deposit formation, catalyst compatibility, and elemental analysis).

deborah amsallem8:37 AM

based on international regulation for engine oil which ppm and elemental molecule can i use. please summarize everything in one table

ChatGPT8:37 AM

I’ll ground this in current lubricant standards/regulatory constraints, then collapse it into one practical table with recommended ppm ranges and validation notes.

Below is the practical regulatory-safe table. Important: API / ILSAC / ACEA do not give an approved list of “marker molecules.” Compliance is controlled through the finished oil limits: SAPS, phosphorus, sulfur, sulfated ash, viscosity, volatility, oxidation, deposits, wear, catalyst/DPF/GPF compatibility. API 1509 governs engine-oil licensing, and ACEA/API control chemical limits such as phosphorus, sulfur and sulfated ash.

| Priority | Element | Market-available molecule | Recommended marker level in finished oil | Regulatory / technical fit | My recommendation |
| --- | --- | --- | --- | --- | --- |
| 1 | Zr | Zirconium neodecanoate / Zr carboxylate | 10–20 ppm Zr | Oil-soluble metal carboxylate, no P/S, low natural background, good replacement for Zr(TMHD)₄. Commercial Zr neodecanoate products exist at ~18% Zr. | Best choice for Shell |
| 2 | Zr | Zirconium 2-ethylhexanoate / octoate | 10–20 ppm Zr | Oil-soluble, commercially available, no P/S. Need SDS review because some products carry solvent/hazard classification. | Good low-cost backup |
| 3 | Ti | Titanium neodecanoate / titanium octoate | 10–20 ppm Ti | Oil-soluble organometallic option, no P/S, low expected impact at ppm level. Less ideal than Zr for Shell because new validation needed. | Possible second element |
| 4 | Y | Yttrium 2-ethylhexanoate / neodecanoate | 5–15 ppm Y | Excellent forensic signature, low background, market available. | Good anti-counterfeit layer |
| 5 | Yb / La / Ce / Nd | Rare-earth neodecanoate / 2-ethylhexanoate | 2–10 ppm element | Very unique elemental code, low natural background, but less automotive history. | Use only after full compatibility testing |
| 6 | Mo | MoDTC / oil-soluble molybdenum additive | 20–80 ppm Mo | Automotive-compatible because MoDTC is already used in engine oils, but it is not inert: it can affect friction, ash and formulation balance. | Not ideal as “neutral marker” |
| Avoid | Ca, Mg, Zn, P, B, Na | Detergent / ZDDP / common additive elements | — | Already present in normal oil additive packages; poor forensic uniqueness. P and S are regulated/catalyst-sensitive. | Avoid |
| Avoid | Fe, Cu, Al, Cr, Ni, Pb, Sn | Wear metals / contaminants | — | Already appear from engine wear or contamination; can create false interpretation in used-oil analysis. ASTM D5185 is commonly used to track additive, wear and contaminant metals. | Avoid |

Best regulatory-safe choice: Zirconium neodecanoate at 10–20 ppm Zr.

Best Shell validation statement:
“The marker is introduced at trace ppm level using an oil-soluble, phosphorus-free and sulfur-free organometallic carboxylate. The final engine oil must remain compliant with the applicable API / ILSAC / ACEA / OEM specification, with no measurable impact on viscosity, oxidation, volatility, foaming, wear, deposits, SAPS, catalyst compatibility or engine performance.”

deborah amsallem8:38 AM

due to background analyiss i cant use zirconium

ChatGPT8:38 AM

If zirconium is excluded because of background levels in virgin oils, additive packages, refinery contamination, or ICP/XRF interference, then I would focus on elements that satisfy all of the following:

- Fully oil-soluble as commercial organometallic compounds

- Not normally present in engine oil additive packages

- Very low natural background in virgin and used oils

- No phosphorus or sulfur contribution

- No known catalyst poisoning concerns at trace ppm levels

- Commercially available from multiple suppliers

- Detectable by ICP-OES, ICP-MS, and XRF

Recommended Elements for Engine Oil Marking

| Rank | Element | Commercial Oil-Soluble Molecule | Recommended Level (ppm element) | Background in Engine Oil | Regulatory Risk | Overall Rating |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Yttrium (Y) | Yttrium neodecanoate / 2-EH | 5–20 ppm | Extremely low | Very low | ⭐⭐⭐⭐⭐ |
| 2 | Ytterbium (Yb) | Yb neodecanoate / 2-EH | 2–10 ppm | Extremely low | Very low | ⭐⭐⭐⭐⭐ |
| 3 | Lanthanum (La) | Lanthanum neodecanoate | 5–20 ppm | Extremely low | Low | ⭐⭐⭐⭐ |
| 4 | Neodymium (Nd) | Neodymium neodecanoate | 5–20 ppm | Extremely low | Low | ⭐⭐⭐⭐ |
| 5 | Cerium (Ce) | Cerium neodecanoate | 5–20 ppm | Low | Medium | ⭐⭐⭐ |
| 6 | Titanium (Ti) | Titanium neodecanoate | 10–20 ppm | Low | Low | ⭐⭐⭐ |
| 7 | Hafnium (Hf) | Hafnium carboxylate | 1–10 ppm | Extremely low | Low | ⭐⭐⭐ |

My Preferred Shortlist for Shell

| Option | Marker |
| --- | --- |
| Best single-element marker | Yttrium (Y) |
| Best forensic marker | Ytterbium (Yb) |
| Best dual-element system | Y + Yb |
| Best triple-element system | Y + La + Yb |

Why Yttrium is Probably the Best Choice

Compared with zirconium:

| Parameter | Zirconium | Yttrium |
| --- | --- | --- |
| Oil-soluble compounds available | Yes | Yes |
| Background in engine oil | Sometimes present | Essentially none |
| Automotive additive interference | Possible | Very low |
| ICP detection | Excellent | Excellent |
| XRF detection | Excellent | Excellent |
| Catalyst concerns | Very low | Very low |
| Anti-counterfeit uniqueness | Medium | Very high |

For a Shell global deployment, I would personally select:

| Element | Concentration |
| --- | --- |
| Yttrium | 10 ppm |
| Ytterbium | 5 ppm |

This gives:

- Extremely low natural background

- Excellent ICP-MS sensitivity

- Excellent XRF detectability

- No phosphorus

- No sulfur

- No zinc

- No calcium

- No interference with used-oil analysis programs

- Strong anti-counterfeit capability through elemental ratio authentication

Elements I Would Not Use

| Element | Reason |
| --- | --- |
| Zn | Existing ZDDP additive |
| Ca | Detergent package |
| Mg | Detergent package |
| P | API/ILSAC regulated |
| S | SAPS regulated |
| B | Increasingly common additive |
| Mo | Friction modifier, affects formulation |
| Cu | Oxidation concerns |
| Fe | Wear metal |
| Ni | Regulatory concerns |
| V | Catalyst poisoning risk |
| Mn | Can affect catalyst systems |

For a premium automotive engine oil intended to remain fully compatible with modern API SP / ILSAC GF-7 / ACEA C-series oils, Yttrium neodecanoate (10 ppm Y) is probably the strongest single-marker candidate currently available from a technical and regulatory perspective.